Digital Signal Processing HW2 110061622 林祐安

1.

For the male voice

(frame length = 0.01 sec)

(frame length = 0.1 sec)

(frame length = 1 sec)

Comparing the above three spectrograms, we can observe that the frame length affects frequency and time resolution. The longer frame length gives rise to some obvious side lobe effects on y-direction due to its high frequency resolution, although we can roughly recognize where the harmonic frequency is. The shorter one results in ambiguous spectrum such that harmonic frequency is difficult to be perceived.

Hence, 0.1 second frame length is worthy to deal with the male voice.

For the female voice

(frame length = 0.01 sec)

(frame length = 0.1 sec)

(frame length = 1 sec)

We encounter the same situation, so about 0.1 second frame length is appropriate for the female voice.

In my perspective, the results generated from 0.05 second to 1 second frame length are able to see the harmonics clearly. Their acceptability depends on the person.

2.

In mathematically, using Hann window instead of rectangular window will get slighter side lobes and a wider main lobe.

(frame length = 0.01 sec male voice)

(frame length = 0.01 sec female voice)

Compare the above two figures to the previous results. The harmonic frequencies generated by Hann window has a wider spread than one from rectangular window as our expected. Hence, if the width of main lobe is too long, harmonics may overlap such that you cannot see separation frequency components. And it affects you to choose frame length.

3.

(female voice with frame length = 0.01 sec and sw.emphasis = 1)

(female voice with frame length = 0.01 sec and sw.emphasis = 0)

To be honest, I cannot even notice any difference between the two figures whereas the comment say it boots up high frequency components. It’s also useless that I also try to change the frame length and voice source.

4.